FWA20P-XGS-32XN Series, 32 Ports XG(S)-PON + CATV

Module EDFA

- Product Description

"Quality Broadband" has become the consensus of the development of the global broadband industry. Meeting the needs of users for 10G bandwidth has become a challenge for operators. To meet the challenges faced by operators, Huawei’s next-generation large-capacity distributed intelligent OLT platform MA5800 provides a single service board that supports XGS-PON, XG-PON, and GPON mixed access application scenarios.

Fullwell is a certified solution partner of Huawei. In order to cater to the integration of triply play network and move towards 10G bandwidth, Fullwell has developed the product of XG (S ) -PON + CATV EDFA high power EDFA. Which its CATV signals and XGS-PON, XG-PON and GPON signals can be accessed on one fiber at the same time, helping operators to create all-optical , automated and intelligent FTTH integrated access network, continuously improve user experience, enable quality broadband, help operators CATV and home broadband become possible, while reducing the cost of operator investment.

FWA20P-XGS series is the latest optical transmission system equipment of XG (S) -PON + CATV FTTH / FTTP triple play network integration, and it is the function of combining OLT XG (S) -PON broadband signal and CATV 1550nm optical signal after amplification. Input ports: 1 CATV + 32 XGS-PON / XG-PON and GPON, combined 32 outputs: 1310 / 1490nm & 1270 / 1577nm + 1550nm, of which the total output power range of 1550nm is 27 ~ 45dBm. Multiply output ports can be customized. Compatible with XG (S) -PON / ONU, such as Huawei, ZTE, and Fiberhome, without losing of data, and has a high cost performance.

2. Product feature

◆ Top quality: Adopt JDSU, Ⅱ-Ⅵ multi-mode high-power pump laser and the power is optimized through software, which makes the system achieving excellent CNR AND minimizing the NF of EDFA.

◆ Reliability: 19 ″ 2U chassis, large-screen with touch screen panel, built-in high-performance hot-pluggable dual switching power supply with Chassis automatic heat dissipation control.

◆ Intuitive: JDSU, Ⅱ-Ⅵ multi-mode pump laser is the most expensive component in the device. The built-in microprocessor monitors the working status of the pump laser, and the working parameters are displayed on LCD.

◆ Network management Interface: Web transponder is line with the national standard and compatible with the SCTE HMS standard, achieving WEB monitoring function.

◆ SNMP Support V1 & V2, support both IPv4 and IPv6.

◆ Customizable 2X1 optical switch: built-in optical path selector, 2 optical inputs, can be set to manual or automatic mode for the selection of main and redundant path.

◆ Integrated WDM of XGS-PON, XG-PON and GPON, optical port: XGS-PON, XG-PON and GPON input:

SC/UPC, 1 port of 1550nm CATV optical input port: SC / APC, XG (S) -PON + CATV 1550nm EDFA output optical port: SC / APC. 64/128 multiply output ports can be customized.

- Technical Parameters

| Items |  | Unit | Performance Index |
| --- | --- | --- | --- |
| Optical Operating Wavelength |  | nm | 1535～1565 |
| Input Optical Power Range |  | dBm | -10～+10 |
| Noise Ratio |  | dB | ≤5.0 (0 dBm,@1550nM) |
| Gain Flatness |  | dB | ＜±0.3 |
| Optical Power Output Stability |  | dB | ＜±0.5 |
| Polarization Sensitivity |  | dB | ＜0.2 |
| Polarization Mode Dispersion |  | Ps | ＜0.5 |
| Input End Pump Leakage Power |  | dB | ≤-30 |
| Output End Pump Leakage Power |  | dB | ≤-30 |
| Optical Input, Output Return Loss |  | dB | ＞45（APC Stepped Face） |
| Pump Operating Number |  | PCs | 2～5 |
| Rated Output Power |  | dBm | 27～45 |
| Linker(IN) |  | - | SC/APC or LC/APC |
| Linker(OUT) |  | - | SC/APC or LC/APC |
| Power Supply/Consumption |  | V/W | AC90～265 or DC-48/20 |
| Operating/Storage Temperature |  | ℃ | -20～65/-30～70 |
| Operating/Storage Humidity |  | % | 5～90 |
| Chassis Dimension |  | mm | 446×245×86（2U） |
| Network Management Connector |  | / | RJ45(following national network management standard, supports WEB) |
|  | XG(S)- PON Wavelength | nm | 1310/1490 & 1270/1577 |
|  | XG(S)- PON Linker | / | SC/UPC or LC/UPC or SC/APC or LC/APC |
|  | Uplink optical ports | pcs | 32 |
|  | Output ports | Pcs | 32 |
|  | XG(S)- PON Insertion Loss | dB | ＜1.2 |
|  | 1550 Port Insertion Loss | dB | ＜0.7 |
|  | Optical isolation between CATV&OLT | dB | 40 |
| Optical Switch | Insertion Loss | dB | ＜1 |
|  | Harass | dB | ＜-60 (isolation between A and B) |
|  | Switch Time | ms | ＜10 |
|  | Switched Power Threshold Range | dBm | -5～10 |

- Product Model Series

| Model Number | Total Output Power dBm(mW) | Output No. | Each Output Power （dBm） |
| --- | --- | --- | --- |
| FWA20P-XGS-16x14 | 28dBm(640mW) | 16 | 14 |
| FWA20P-XGS-32x10 |  | 32 | 10 |
| FWA20P-XGS-16x15 | 29dBm(800mW) | 16 | 15 |
| FWA20P-XGS-32x11 |  | 32 | 11 |
| FWA20P-XGS-16x16 | 30dBm(1000mW) | 16 | 16 |
| FWA20P-XGS-32x12 |  | 32 | 12 |
| FWA20P-XGS-16x17 | 31dBm(1300mW) | 16 | 17 |
| FWA20P-XGS-32x13 |  | 32 | 13 |
| FWA20P-XGS-16x18 | 32dBm(1600mW) | 16 | 18 |
| FWA20P-XGS-32x14 |  | 32 | 14 |
| FWA20P-XGS-16x19 | 33dBm(2000mW) | 16 | 19 |
| FWA20P-XGS-32x15 |  | 32 | 15 |
| FWA20P-XGS-16x20 | 34dBm(2500mW) | 16 | 20 |
| FWA20P-XGS-32x16 |  | 32 | 16 |
| FWA20P-XGS-16x21 | 35dBm(3000mW) | 16 | 21 |
| FWA20P-XGS-32x17 |  | 32 | 17 |
| FWA20P-XGS-16x22 | 36dBm(4000mW) | 16 | 22 |
| FWA20P-XGS-32x18 |  | 32 | 18 |
| FWA20P-XGS-16x23 | 37dBm(5000mW) | 16 | 23 |
| FWA20P-XGS-32x19 |  | 32 | 19 |
| FWA20P-XGS-32x20 | 38dBm(6000mW) | 32 | 20 |
| FWA22P-XGS-64x16 |  | 64 | 16 |
| FWA20P-XGS-32x21 | 39dBm(8000mW) | 32 | 21 |
| FWA22P-XGS-64x17 |  | 64 | 17 |
| FWA20P-XGS-32x22 | 40dBm(10000mW) | 32 | 22 |
| FWA22P-XGS-64x18 |  | 64 | 18 |
| FWA20P-XGS-32x23 | 41dBm(13000mW) | 32 | 23 |
| FWA22P-XGS-64x19 |  | 64 | 19 |