Specification for Approval

Product Name     Standard L2 Managed PoE Switch

Model            Hi-G2444GBM

Version           A1.0

Content

1. The description of product	3

2.The feature of product	3

3.The index of product property	3

3.1 The electrical feature	3

3.2 The property of switch network	3

3.3 The software feature	4

3.4 The property of PoE power supply	6

4.Safety Requirements	6

5.Electromagnetic Compatibility	7

5.1 Electromagnetic Interference EMI	7

5.2 Electromagnetic Compatibility	7

6.Insulation Performance and Earth Resistance	8

7. Environmental requirements	8

7.1 Environmental temperature	8

7.2 Humidity	8

7.3 Altitude	8

7.4 Cooling Method	8

7.5 Vibration Resistance	9

7.6 Shock Resistance	9

8.Interface Definition	9

9.Weight	10

10. Product Size	10

11.Packing List	10

12.Packaging, Shipping, Storage	11

12.1 Packaging	11

12.2 Transport	11

12.3 Storage	11

1. The description of product

Hi-G2444GBM L2 Managed PoE switch, it is equipped with high-performance hardware and software platform, provides 24*1000Mbps PoE ports, 4*1000Mbps Uplink ports and 4*1000Mbps SFP ports (Combo port). The PoE port with 802.3af/at standard, the maximum power of the whole machine is 400W. It adopts 4M large cache chip, support L2 management, such as VLAN, QoS, IGMP Snooping and PoE management.

2. The feature of product

24*1000Mbps PoE ports

4*1000Mbps Uplink ports

4*1000Mbps SFP ports (Compo port)

1* Console port

Support L2 management, such as VLAN, QoS, IGMP Snooping and PoE management.

WEB, TELNET, CLI management

Port lightning protection 6KV

3. The index of product property

3.1 The electrical feature

List 1: The electrical feature

| No. | Project | Minimum | Regular | Maximum | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Input Voltage | 110 | 220 | 240 | Vac |  |
| 2 | Input Frequency | 47 | 50 | 63 | Hz |  |
| 3 | Input Current |  |  | 4.5 | A |  |
| 4 | Standby Power Consumption |  |  | 18 | W | Camera Not Connected or Loaded |
| 5 | Electric Shock |  |  | 100 | A | AC input 110Vac，Cold Start |

3.2 The property of switch network

List 2: Network feature

| No. | Project | Index of Performance |
| --- | --- | --- |
| 1 | Network Standard | IEEE802.3、IEEE802.3u、IEEE802.3x、 IEEE802.3ab、IEEE802.1Q、 IEEE 802.1p |
| 2 | Network Medium | 10/100/1000BASE-TX: 5 class and above non shielded twisted pair |
| 3 | Data Distance | ≤250M |
| 4 | Switch Capability | 56Gbps |
| 5 | Forwarding Rate | 10BASE-tx: 14880pps/port 100BASE-tx: 148800pps/port 1000BASE-tx: 1488000pps/port |
| 6 | MAC Address | MAC address table 8K |
| 7 | Port Function | Power priority mechanism, fast and forward, MAC automatic learning and aging IEEE802.3X Full-duplex and mode and backpressure for Half-duplex mode |
| 8 | Forwarding Rate of Packet | 41.7Mpps |

3.3 The software feature

List 3: Software feature

| No. | Function Item |  |
| --- | --- | --- |
| 1 | Web management | Default IP: 192.168.2.1 User：admin Password：admin |
| 2 | Management IP | Supports port enable and disable Support port rate setting Support port mirroring Supports port isolation |
| 3 | VLAN | Port-based VLANs, ACCESS VLANs, TRUNK VLANs, HYBRID VLANs, 802.1Q tagged VLANs, supports up to 4094 VLANs |
| 4 | MAC management | MAC Address Display/Inquiry Static MAC Address Setting Dynamic MAC Address Management MAC Address Filtering |
| 5 | Security Features | Port isolation within VLAN Port+MAC+IP binding Support Rapid Spanning Tree Protocol (RSTP) |
| 6 | PoE Management | PoE status query Supports PoE timer setting |
| 7 | QoS | Support WRR DSCP Priority IP Priority |
| 8 | System Management and Maintenance | Support WEB management and telnet configuration. Support CLI configuration. Support checking CPU and memory usage. Message statistics |

3.4 The property of PoE power supply

List 4: The property of PoE power supply

| No. | Project | Index of Performance |
| --- | --- | --- |
| 1 | PoE Protocol | IEEE 802.3af/at |
| 2 | Output Power of PoE Port | Single Port≤30W |
| 3 | PoE Output Voltage | DC52V±5% |
| 4 | PoE Power Pin | 1/2+、3/6- |
| 6 | PoE Power Distance | ≤250m |
| 7 | Network Medium | 10/100/1000BASE-TX: 5 class and above non shielded twisted pair |
| 8 | Whole Power | MAX: 400W |

4.Safety Requirements

List 5: Safety requirements sheet

| Project | Technical Index | conform or not | Note |
| --- | --- | --- | --- |
| Safety | UL60950(60065) | Yes No |  |
|  | IEC/EN60065(60950) | Yes No |  |
|  | GB4943 | Yes No |  |

5.Electromagnetic Compatibility

5.1 Electromagnetic Interference EMI

List 6: EMI

| Project | Technical Index | conform or not | Note |
| --- | --- | --- | --- |
| Switch AC Input Port |  |  |  |
| Conducted disturbance | GB9254, CLASS B | Yes No |  |
|  | FCC PART15 CLASSB | Yes No |  |
|  | EN55032 | Yes No |  |
| Radiated interference | GB9254, CLASS B | Yes No |  |
|  | FCC PART15 CLASSB | Yes No |  |
|  | EN55032 | Yes No |  |

5.2 Electromagnetic Compatibility

List 7: EMS

| Project | Technical Index | conform or not | Note |
| --- | --- | --- | --- |
| Switch AC Input Port |  |  |  |
| Static immunity | GB17626.2-1998 | Yes No | Contact discharge 4KV, air discharge 8KV |
|  | IEC61000-4-2 | Yes No | Contact discharge 4KV, air discharge 8KV |
| Burst immunity | GB17626.4-1998 | Yes No |  |
|  | IEC61000-4-4 | Yes No |  |
| Lightning surge | GB17626.4-1998 | Yes No | Common mode: 4KV, Differential mode: 3KV |
|  | IEC61000-4-4 | Yes No | Common mode: 4KV, Differential mode: 3KV |
| Switch Uplink Port |  |  |  |
| Static immunity | GB17626.2-1998 | Yes No | Contact discharge 4KV, air discharge 8KV |
|  | IEC61000-4-2 | Yes No | Contact discharge 4KV, air discharge 8KV |
| Lightning surge | GB17626.4-1998 | Yes No | Common mode: 6KV, Differential mode: 1KV |
|  | IEC61000-4-4 | Yes No | Common mode: 6KV, Differential mode: 1KV |
| Switch PoE Port |  |  |  |
| Static immunity | GB17626.2-1998 | Yes No | Contact discharge 4KV, air discharge 8KV |
|  | IEC61000-4-2 | Yes No | Contact discharge 4KV, air discharge 8KV |
| Lightning surge | GB17626.4-1998 | Yes No | Common mode: 6KV |
|  | IEC61000-4-4 | Yes No | Common mode: 6KV |

6.Insulation Performance and Earth Resistance

List 8: Insulation resistance

| AC L/N to ground | DC 500V ≥30M |
| --- | --- |

List 9: Insulation withstand voltage

| AC L/N to ground | AC 1500V ,1MIN, <10mA |
| --- | --- |

List 10: Ground resistance

| Chassis to ground | 32A ,2MIN, <0.01Ω |
| --- | --- |

7. Environmental requirements

7.1 Environmental temperature

*Working: -10℃-55℃

*Storage: -40℃-85℃

7.2 Humidity

*Working: 10% to 90% relative humidity (non-condensing)

*Storage: 5% to 95% relative humidity (non-condensing)

7.3 Altitude

*Working: Up to 6500ft (2000m)

*Storage: Up to 6500ft (2000m)

7.4 Cooling Method

Ventilation cooling

7.5 Vibration Resistance

*10-55Hz, 49.0m/s2(5G), cycle 3 minutes, 60 minutes each for X, Y, Z axis.

7.6 Shock Resistance

*196.1m/s2 (20G), 11ms, once each for X, Y and Z axes

8.Interface Definition

Picture 8: Definition of interface

9.Weight

Bare metal: 3750g±5%

Finished product: 3150g±5%

10. Product Size

Dimensions of Bare Metal: (Length)440mm*(Width)284mm*(High)44mm

Finished Product Size: (Length)570mm*(Width)360mm*(High)90mm

11.Packing List

List 11: Packing List

| No. | Packing List | Quantity | Unit |
| --- | --- | --- | --- |
| 1 | User manual | 1 | PCS |
| 2 | Hanger ear set | 1 | SET |
| 3 | International power Line | 1 | PCS |
| 4 | PoE Switch | 1 | PCS |

12.Packaging, Shipping, Storage

12.1 Packaging

The packing box should show the product name, model, manufacturer's logo, the inspection certificate of the manufacturer's quality department, and the date of manufacture.

12.2 Transport

It is suitable for transportation by car, ship and airplane. It should be covered with awning, sun protection and handled with care during transportation.

12.3 Storage

It should be put in the packing box when the product is not in use, the temperature of the warehouse is -40℃-+85℃, and the relative humidity is 5%--95%. It is not allowed in the warehouse that there is harmful gas, flammable and explosive products and other corrosive chemicals products, and not allowed that there is strong mechanical vibration, shock and strong magnetic field. The packing box should be at least 20mm above the ground, and at least 50cm away from walls, heat sources, windows or air inlets. Generally, it can be stored in 2 years when under this storage condition, more than 2 years should be re-inspected.